# Engineering Units Conversion and Calculation Specification

Gary Choquette

May 11, 2026

# Background

In engineering calculations, it is common for different engineering units to be used in different countries. There are multiple issues when calculating with engineering units including:

- Mixed units of the same type such as 6-feet, 5-inches,

- Multiplying units of mixed type to produce intermediate units such as 55 lbf * 5 ft / 0.5 s = 1 Hp,

- ‘Standardized’ formulas that used ‘non-standard’ base (e.g. SI) units,

- Specify preferred units,

- Specify the number of units digits of precision,

- Performs ‘overloaded math’ of

- Industry ‘common’ units that are not based on standard base units (e.g., MMSCFD) and

- Displaying values in professional documents (reports, conference papers, etc.) in both US customary and SI customary values.

  - Use .ToString to automatically format the output both with respect to precision and engineering units. Optionally allow ‘long format’ units (e.g. ‘feet’ rather than ‘ft’).

# Specification

Develop a Visual Studio class library in VB or C# that performs engineering unit calculations. The user specifies their ‘preferred units’, if none are specified the default is SI. Each fundamental unit type shall be represented (e.g., length, time, mass, rotation) and common ‘derived’ units as well (e.g., area, volume, volume flow rate, velocity, etc.). The user specifies the value and the engineering units (using a convention to be defined, for example ft-s-1 could represent feet per second, ft3/s could be cubic feet per second. As many engineering units for each unit type shall be included (e.g., furlongs, miles, nautical miles,

When two values of the same unit type are added/subtracted together but they are of mixed units of the same unit type, the output will use the engineering units of the first value in the equation. For example, if A is a length unit in ft (feet) of value 5 and B is a length unit in in (inches) of value 6, C = A + B yields C = 5.5 ft. C = B + A yields C = 66 in. Attempts to add/subtract units of different types (length + time) result in an error.

Multiplication/division results in ‘derived’ units; for example, multiplying a length unit by an area unit produces a volume unit.

## Dimensionless units

Some engineering calculations result in dimensions numbers such as Mach Number, Cowling Number, Euler Number, Eckert Number, Jakob Number, etc. When the calculation results in an obvious dimensionless unit, the engineering unit should state the corresponding dimensionless unit. Note that not all velocity ratios are Mach numbers so when the corresponding dimensionless number is not clear (as perhaps the default in most cases), the default unit would be the unreduced engineering units (e.g., ft-s-1/ft-s-1). The user could then select a ‘preferred’ dimensionless unit (e.g., Mach Number, Velocity Ratio).

# Resources

https://www.optimizedtechnicalsolutions.com/ReferenceDocuments/EngineeringUnits.xls

https://www.iist.ac.in/sites/default/files/2025-06/dimensionless.pdf

https://www.unitconverters.net/